📄 ESP32 Gas Monitoring System (MQ-2 + Firebase + Live Dashboard)

🧠 Project Overview

This project is an IoT-based gas monitoring system using ESP32 and MQ-2 sensor. It sends real-time gas data to Firebase and visualizes it using a live web dashboard hosted on Firebase Hosting.

The system went through multiple debugging stages, including WiFi issues, serial communication problems, Firebase misconfiguration, and deployment confusion. This documentation includes both correct solutions and mistakes made during development.

⚙️ System Architecture

ESP32 + MQ-2 Sensor
        ↓ (WiFi)
Firebase Realtime Database
        ↓
Firebase Hosting Web Dashboard
        ↓
Live Graph (Chart.js)

🧰 Hardware Used

- ESP32 development board

- MQ-2 gas sensor

- USB cable

- WiFi connection

🧪 Software Stack

- Arduino IDE

- Firebase Realtime Database

- Firebase Hosting

- HTML + JavaScript

- Chart.js

- HTTPClient (ESP32)

🟢 FINAL WORKING FLOW

Sensor → ESP32 → Firebase → Web Dashboard → Live Graph

❌ PROBLEMS FACED (REAL DEBUGGING JOURNEY)

❌ Problem 1: Serial Monitor completely empty

Symptoms:

- No output in Serial Monitor

- ESP32 powered (blue LED blinking)

Wrong assumptions:

- Code issue

- Sensor issue

Root cause:

- Wrong COM port selected

- Incorrect baud rate (not matching 115200)

- USB cable sometimes faulty

Solution:

✔ Set baud rate to 115200
✔ Select correct COM port
✔ Press EN reset button
✔ Change USB cable when needed

❌ Problem 2: ESP32 WiFi connection confusion

Issue:

WiFi credentials changed but device not responding.

Mistake:

- Assuming code was broken

Solution:

✔ Fixed SSID/password
✔ Verified connection loop:

while (WiFi.status() != WL_CONNECTED)

❌ Problem 3: Firebase URL confusion

Mistake:

Confusing Firebase console URL with database URL.

Wrong URLs tried:

- Firebase console link ❌

- Hosting docs link ❌

Correct URL format:

https://project-default-rtdb.asia-southeast1.firebasedatabase.app/

Solution:

✔ Used correct Realtime Database URL
✔ Added .json for REST API calls

❌ Problem 4: Data not appearing in Firebase

Mistake:

Using wrong method (PUT instead of POST for history)

Result:

Only single value overwritten repeatedly

Solution:

✔ Switched to:

- POST → for history data

- /gas_readings.json → correct endpoint

Now Firebase stores:

gas_readings:
  -Nabc1: { gas: 420 }
  -Nabc2: { gas: 430 }

❌ Problem 5: Firebase Hosting showed documentation page

Symptoms:

Instead of dashboard, Firebase showed:

- Firebase docs page

Wrong assumptions:

- Firebase broken

- Hosting not working

Real cause:

- Incorrect deployment structure OR cached domain issue

Solution discovered:

✔ Correct domain used:

https://esp32-gas-monitor-f8320.firebaseapp.com

✔ web.app domain was inconsistent due to caching/propagation delay

❌ Problem 6: Firebase Hosting setup confusion (CLI)

Mistake:

- Running firebase init in wrong directory (Desktop)

- Not understanding folder structure

Wrong structure attempt:

Desktop/dashboard.html

Correct structure:

project/
  public/
    index.html

Solution:

✔ Created proper project folder
✔ Used public/index.html
✔ Redeployed correctly

❌ Problem 7: Dashboard not loading Firebase data

Symptoms:

- Graph empty

- No live updates

Causes:

- Running HTML via file:// protocol

- Browser blocking Firebase requests

Solution:

✔ Hosted via Firebase Hosting
✔ Used proper HTTPS domain

🚀 FINAL WORKING SYSTEM

✔ ESP32 Code Responsibilities

- Reads MQ-2 sensor

- Sends gas data via WiFi

- Pushes data to Firebase

✔ Firebase Responsibilities

- Stores real-time gas data

- Stores history (time-series data)

✔ Web Dashboard Responsibilities

- Fetches Firebase data

- Displays live graph using Chart.js

- Updates every 2 seconds

📈 FINAL FEATURES ACHIEVED

✔ Real-time gas monitoring
✔ Cloud-based IoT system
✔ Live graph dashboard
✔ Data history storage
✔ Mobile/remote access
✔ Firebase Hosting deployment

🧠 KEY LESSONS LEARNED

1. IoT debugging is multi-layered

- Hardware → Serial → WiFi → Cloud → Frontend

2. Most issues were NOT code bugs

- Folder structure mistakes

- Wrong URLs

- Browser caching

- Deployment mistakes

3. Firebase has multiple systems

- Realtime DB (data)

- Hosting (frontend)

- Each behaves differently

🔥 FUTURE IMPROVEMENTS

📱 Mobile App

- Flutter-based dashboard

🚨 Gas Alert System

- Threshold detection

- Buzzer + notifications

📡 MQTT Upgrade

- Multi-device communication

🏭 Industrial Dashboard

- Authentication system

- Multi-sensor monitoring

- Analytics layer

🏁 FINAL SUMMARY

This project evolved from:

Basic ESP32 sensor reading
        ↓
WiFi debugging issues
        ↓
Firebase integration problems
        ↓
Hosting & deployment confusion
        ↓
Fully working IoT cloud system